User Guide:

Azure IoT Hub Device Update for the WFI32-IoT Development Board

Table of Contents

Overview	3

Prepare Azure Resources	5

Create an IoT Hub	5

Register a Device with the IoT Hub	7

Create Device Update Account and Instance	8

Prepare the New Firmware	9

Clone/Download the Example Project	9

Build New Firmware	9

Prepare New Firmware for the Simulated Leaf Device	12

Generate Import Manifest	12

Upload Firmware and Manifest	13

Prepare the Device	16

Configure Wi-Fi Settings	16

Revert to the Previous Firmware Version	17

Program and Run the Device	17

Deploy New Firmware	19

Add a Tag to Your Device	19

Deploy New Firmware Update	19

Clean Up Resources	26

Next Steps	27

0

# Overview

The following steps detail how to configure, build, and execute the Device Update for IoT Hub example on the Microchip WFI32-IoT Development Board. For this example, we will learn how to:

- Create Azure IoT Hub and Device Update for IoT Hub resources

- Prepare the firmware for the dev board and the required manifest file

- Prepare the firmware for the simulated leaf device for proxy update and the required manifest file

- Upload the files and create the group for the device in the Device Update for IoT Hub

- Deploy the firmware to the device and observe the results

The IDE that was used testing this example is MPLAB X version 6.05. You will also need MPLAB XC32 compiler version 4.10 to build the example project. Earlier versions of Microchip’s MPLAB X IDE and XC compilers can be found in the MPLAB Development Ecosystem Downloads Archive.

Figure 1 Microchip WFI32-IoT Development Board

# Prepare Azure Resources

## Create an IoT Hub

You can use the Azure Command Line Interface (CLI) to create an IoT hub that handles events and messaging for your device.

CLICK

Log into your Azure portal account and click on the Cloud Shell icon located at the top of the web page (you may be prompted to create a storage account in order to use Cloud Shell)

CLICK

In the upper left-hand corner of the Cloud Shell pane, confirm that PowerShell is selected

- At the PS prompt, run the az extension add command to add the Microsoft Azure IoT Extension for Azure CLI to your CLI shell. The IoT Extension adds IoT Hub, IoT Edge, and IoT Device Provisioning Service (DPS) specific commands to the Azure CLI.

az extension add --name azure-iot

- Run the az account list command to see the list of subscriptions accessible by your Azure account

az account list

- Run the az account set command to select the specific subscription you want to use for creating the Azure resources

az account set --subscription "{MySubscriptionName}"

- Run the 1 (or 2) command(s) required to create a resource group. For example, if your company’s policy requires specific tags be added to a resource group, you can create a “tags” variable to point to a list of all the tag name/value pairs required before creating the resource group. Otherwise if no tags are needed, just run the New-AzResourceGroup command and without the -tag option. To set a specific location for your resource group, run az account list-locations to see a list of all available regions.

$tags=@{"name1"="val1";"name2"="val2";"name3"="val3”;etc}

New-AzResourceGroup -Name {MyResourceGroup} -Location {MyLocation} [-tag $tags]

For example, the following 2 command lines will create a resource group named ADU_TestGroup with a location of westus along with 8 different tags which might be used for a company’s policy for accounting purposes:

$tags=@{"BusinessOwner"="Rob Stein - C07707"; "CostCenter"="SESE00"; "CreatedBy"="Randy Wu - C14166"; "CreationDate"="20230421"; "Env"="DEV"; "Group"="N/A"; "ManagedBy"="Randy Wu - C14166"; "SNOWTicket"="N/A"}

New-AzResourceGroup -Name ADU_TestGroup -Location westus -tag $tags

NOTE: If using Bash (UNIX) CLI, execute the following command line:

az group create --name {MyResourceGroup} --location {MyLocation} [--tags name1="value1" name2="value2" name3="value3" etc...]

…for example:

az group create --name ADU_TestGroup --location westus --tags BusinessOwner="Rob_Stein - C07707" CostCenter=SESE00 CreatedBy="Randy Wu - C14166" CreationDate=20230421 Env=DEV Group=N/A ManagedBy="Randy Wu - C14166 SNOWTicket=N/A

- Run the az iot hub create command to create an IoT hub. It might take a few minutes for Azure to finalize the creation of the IoT hub…

Replace YourIotHubName below with the name you choose for your IoT Hub. An IoT Hub name must be globally unique in Azure.

az iot hub create --resource-group {MyResourceGroup} --name {YourIoTHubName}

… for example:

az iot hub create –resource-group "ADU-TestGroup" --name "ADU-IoTHub"

- After the IoT hub is created, view the JSON output in the console, and copy the hostName value to a safe place. You will use this value in a later step. The hostName value will be in the following format:

{YourIoTHubName}.azure-devices.net

[TIP] You can look up the host name again by using the following command to list all the IoT Hub host names that are tied to your portal account:

az iot hub list --query "[].{hostname:properties.hostName}" --output table

## Register a Device with the IoT Hub

In this section, you create a new device instance and register it with the IoT Hub you created. You will use the connection information for the newly registered device to securely connect your physical device in a later section.

- In your console, run the az iot hub device-identity create command. This creates the simulated device identity.

YourIotHubName. Replace this placeholder below with the name you chose for your IoT hub.

MyDevKit. You can use this name directly for the device in CLI commands in this tutorial. Choose a meaningful name for your kit (e.g. WFI32-IoT)

az iot hub device-identity create --device-id "{MyDevKit}" --hub-name "{YourIoTHubName}"

… for example:

az iot hub device-identity create --device-id "WFI32-IoT" --hub-name "ADU-IoTHub"

- After the device is created, view the JSON output in the console, copy and save the deviceId and primaryKey values to use in a future step.

Confirm that you have the copied and saved the following values from the JSON output to use in the next section:

- hostName

- deviceId

- primaryKey

TIP: You can get the primaryKey at any time by executing the following command in the Azure CLI:

az iot hub device-identity show --hub-name {YourIoTHubName} --device-id {MyDevKit}

… for example:

az iot hub device-identity show --hub-name "ADU-IoTHub" --device-id "WFI32-IoT"

## Create Device Update Account and Instance

Click on one of the following hyperlinks to create a Device Update account and instance using one of the following methods:

- Azure portal

- Azure CLI

# Prepare the New Firmware

To connect the device to Azure, you'll modify a configuration file for Azure IoT settings, build the example project, and flash the image to the device.

## Clone/Download the Example Project

Using the Git command line or Desktop tool, clone the repository located at the URL specified in the following command line:

git clone https://github.com/MicrochipTech/AzureDemo_DeviceUpdate

As an alternative, you can simply go to the specified URL and download a ZIP file of the repository:

2

1

## Build New Firmware

- Launch the MPLAB X IDE and select File > Open Project from the main toolbar. Navigate to the example project folder by browsing to AzureDemo_DeviceUpdate > WFI32-IoT > AzureDemo_WFI32E01 > firmware > AzureDemo_WFI32-IoT.X

- In the Projects pane, right-click on AzureDemo_WFI32-IoT and select Set as Main Project.

- In the Projects pane, expand the main Header Files folder and navigate to the azure_rtos_demo > sample_azure_iot_embedded_sdk_pnp sub-folder.

- Double-click the sample_config.h header file to set the Azure IoT device information constants to the values that you copied and saved after you created Azure resources in a previous section.

Double-click to edit

| Constant name | Value |
| --- | --- |
| HOST_NAME | {Your IoT Hub hostName value} |
| DEVICE_ID | {Your deviceID value} |
| DEVICE_SYMMETRIC_KEY | {Your primaryKey value} |

…for example:

Originally 1.0.0

In the Projects pane, expand the Source Files folder and navigate to the azure_rtos_demo > sample_azure_iot_embedded_sdk_pnp sub-folder. Double-click on sample_azure_iot_embedded_sdk_adu.c to modify the firmware version (e.g. 1.0.0 to 1.1.0) to mimic the need for a new firmware deployment from Azure Device Update.

#define SAMPLE_DEVICE_INSTALLED_CRITERIA "1.1.0"

Originally 1.0.0

#define SAMPLE_LEAF_DEVICE_INSTALLED_CRITERIA "1.1.0"

- In the Projects pane, right-click on the AzureDemo_WFI32-IoT project and select the Clean and Build option. Wait for the project to finish building and confirm that the build was successful.

NOTE: XC32 compiler version 4.10 is required for a successful build; higher versions may result in compiler errors.

- Open a command line window (Command Prompt, PowerShell, Bash) and navigate to the folder AzureDemo_DeviceUpdate > WFI32-IoT > AzureDemo_WFI32E01 > tools > hex2bin

- Execute the following command line (for MacOS users only):

> pip3 install colorama

- Execute one of the following command lines based on your Operating System to convert the project’s HEX (*.hex) file into a binary (*.bin) file

[MAC]

> python3 hex2bin.py -i ../../firmware/AzureDemo_WFI32-IoT.X/dist/pic32mz_w1/production/AzureDemo_WFI32-IoT.X.production.unified.hex -o AzureDemo_WFI32-IoT.X.production.unified.bin

[WINDOWS/LINUX]

> hex2bin.exe -i ../../firmware/AzureDemo_WFI32-IoT.X/dist/pic32mz_w1/production/AzureDemo_WFI32-IoT.X.production.unified.hex -o AzureDemo_WFI32-IoT.X.production.unified.bin

- Rename the binary file to “firmware_<version>.bin” (e.g. “firmware_1.1.0.bin”)

- Copy the newly-named binary file to the folder AzureDemo_DeviceUpdate > WFI32-IoT > AzureDemo_WFI32E01 > tools > AzureDeviceUpdateScripts

You have just prepared the new firmware (binary file) that will be uploaded to the “Device Update for IoT Hub” service.

## Prepare New Firmware for the Simulated Leaf Device

Even though we do not physically connect any leaf devices to the dev board we are using, it is a common scenario for your MCU device that you might have one or more leaf devices connect to it via UART or USB as peripherals or sensors. For this example, we just use a simulated leaf device and push an empty firmware to demonstrate the experience. To do that:

Create a file and name it leaf_firmware_1.1.0.bin and save it in the same AzureDemo_DeviceUpdate > WFI32-IoT > AzureDemo_WFI32E01 > tools > AzureDeviceUpdateScripts folder with this PowerShell command:

New-Item "leaf_firmware_1.1.0.bin" -ItemType File -Value "Empty leaf firmware"

## Generate Import Manifest

An import manifest is a JSON file that defines important information about the update that you are importing that is required by the Device Update for IoT Hub. You can learn about the detailed steps of importing a new update here. For this example:

- Ensure you have installed PowerShell v7.0 or higher.

- In a PowerShell window, navigate to the AzureDemo_DeviceUpdate > WFI32-IoT > AzureDemo_WFI32E01 > tools > AzureDeviceUpdateScripts directory and run the command:

Set-ExecutionPolicy -ExecutionPolicy Bypass -Scope Process

- Run the Python script and then enter info when prompted to generate the manifest files for the dev board and the simulated leaf device:

.\CreateWFI32Update.ps1

Supply values for the following parameters:

(Type !? for Help.)

Version: 1.1.0

HostPath: ./firmware_1.1.0.bin

LeafPath: ./leaf_firmware_1.1.0.bin

- Upon successful completion of the script, all files required (to upload into the Device Update service) is generated in the folder named MICROCHIP.WFI32.1.1.0

## Upload Firmware and Manifest

- Access your Azure portal account to find the IoT Hub resource which was created earlier. You may have to first access the resource group that contains it in order to find the IoT Hub. When the IoT Hub has been found in your Azure portal, click on the IoT Hub name to access its configuration details.

- Using the left-hand navigation pane, select Device Management > Updates

CLICK

- Locate the Updates tab and then select Import a new update

CLICK

- Choose + Select from storage container. If no storage accounts show up in the list, create a new one now by selecting + Storage account.

- From the list of Storage accounts, click on the name of the newly-created (or pre-existing) storage account that you would like to use for storing the manifest files and binary images required for the Device Update process.

- You should now be on the Containers page. If no containers show up in the list, select + Container and enter in a name for the new container. Click on the Create button.

- In the list of existing Containers, click on the name of the container that you would like to use for storing the Device Update files.

- Select Upload and browse to (or drag-and-drop) all of firmware image and manifest files required for the deployment from the MICROCHIP.WFI32.1.1.0 folder.

- Click on the Upload button. You should see the message “Successfully uploaded blob(s)” and then see a list of all the uploaded files (4 blobs total):

Now you have the new firmware images and manifest files uploaded into a container, which prepares you for creating a deployment in a future section. It’s now time to turn our attention to the device and get it up and running to receive the new firmware.

# Prepare the Device

We will build and run the same device application but for a previous version, so that later we can observe the new firmware being deployed to update to the newer version.

## Configure Wi-Fi Settings

- Connect a micro-USB cable to the USB connector on the Microchip WFI32-IoT Development Board, and then connect the other end to your computer’s USB port.

- Using a File Explorer window (or Desktop view), confirm that a Mass Storage Device (most likely named CURIOSITY) appears.

- Click here and use the web-based configurator tool to configure the WFI32-IoT Development Board's device settings with your wireless router’s SSID, network type, and password. To be on the safe side, there should be no spaces used in the SSID and password for your network, and the Wi-Fi Access Point should be operating in the 2.4 GHz frequency band.

## Revert to the Previous Firmware Version

- In the Projects pane of the MPLAB X IDE, expand the Source Files folder and double-click on the source file sample_azure_iot_embedded_sdk_adu.c and modify the firmware version number back to the original:

#define SAMPLE_DEVICE_INSTALLED_CRITERIA "1.0.0"

#define SAMPLE_LEAF_DEVICE_INSTALLED_CRITERIA "1.0.0"

- In the Projects pane, right-click on the AzureDemo_WFI32-IoT project and select Clean and Build. Verify that the build process is successful before proceeding.

## Program and Run the Device

- Confirm that the WFI32-IoT Development Board is connected to the computer via the micro USB cable.

CLICK

Click the Make and Program Device icon in the main toolbar.

- Launch your OS’s device manager and verify the serial port connection corresponding to the WFI32-IoT Development Board’s USB connection – it should show up as a Virtual COM port:

- Open your favorite serial terminal program such as Putty or Tera Term and connect to the COM port number discovered in the device manager (use 115200-8-N-1).

- Press the RESET button on the WFI32-IoT Development Board.

- As the demo project runs on the device, it continuously prints out status information to the terminal output window. Check the terminal output to verify that there are no error messages and that there is a message stating that the device has “Connected to IoTHub”. Note the output message towards the end that displays the installed criteria version number.

NOTE: The terminal output content may vary depending on which example project you choose to build and run. Keep the terminal window open to monitor device output in subsequent steps.

# Deploy New Firmware

## Add a Tag to Your Device

- Confirm that the device application is still running from the previous step (i.e. normal execution was never interrupted).

- Log into the Azure portal and navigate to the IoT Hub (you may need to access the resource group that’s tied to the IoT Hub in order to view it).

- Using the left-hand navigation pane, select Device Management > Devices.

- Your device ID should show up in the list of devices; click directly on its name in the list.

- Click on the Device Twin tab.

- Create a deployment group by adding a tag to the Device Twin:

"tags": {

"ADUGroup": "{MyDeploymentGroupName}"

}

NOTE: To remove any tag in the future, the tag value can be set to ‘null’

- Click on the Save icon in the upper left-hand corner of the page, then click on the X in the upper right-hand corner of the page to close the Device twin view.

## Deploy New Firmware Update

- In the Azure portal, navigate to the IoT Hub that you previously connected to your Device Update instance (you may need to navigate to the resource group which contains the IoT Hub in order to access it).

- Using the left-hand navigation pane, select Device Management > Updates

- Select + Import a new update.

- Select + Select from storage container.

- Click on the name of the blob storage which contains the container that stores the manifest files and binary images.

- Click on the container that stores the manifest files and binary images.

- Check the boxes for all the files in the list and then click the Select button at the bottom of the page.

1

2

- Confirm that the manifest and binaries show up on the resulting page. Click the Import update button at the bottom of the page.

CLICK

- You may get a warning message regarding the status of the import; ignore the message and proceed with the next step.

- Confirm that a new update was created showing the correct firmware version number. It may take a minute or two for the update to appear in the list, so click on the Refresh icon periodically until the WFI32 update shows up in the list.

CLICK

- Click on the Groups and Deployments tab.

CLICK

The name of the deployment group that you just created (e.g. “TestDevices”) should show up under the list of device groups. Click on the Deploy link which should show up under the Status column for the deployment group.

- The New Updates pop-up window should appear, showing the correct firmware version for deployment. Click on the Deploy button.

CLICK

- The Create deployment pop-up window should appear. Leave the default option as Start immediately and click on the Create button to start the deployment.

CLICK

- Go back to observing the terminal output window. You can see the update firmware is pushed from ADU to the device. After completing the upload, the device will reboot with the new firmware:

Since the simulated leaf device just does nothing, the terminal output will only show that the empty firmware is deployed to the device and device will echo back to it.

- To cancel the deployment, click on the Groups and Deployments tab and then locate the deployment group (e.g. “TestDevices”) under the list of device groups. Click on the Deploy link which should show up under the Status column for the deployment group.

CLICK

- Locate the name/version of the device update and click on its View button.

CLICK

- On the Current updates tab, click on the Cancel deployment button.

CLICK

# Clean Up Resources

If you no longer need the Azure resources created in this tutorial, you can use the Azure CLI to delete the resource group and all the resources you created for this tutorial. Optionally, you can use Azure IoT Explorer to delete individual resources including devices and IoT hubs.

If you continue to another tutorial in this getting started guide, you can keep the resources you've already created and reuse them.

Important: Deleting a resource group is irreversible. The resource group and all the resources contained in it are permanently deleted. Make sure that you do not accidentally delete the wrong resource group or resources.

To delete a resource group by name:

- Run the az group delete command. This removes the resource group, the IoT Hub, and the device registration you created.

az group delete --name {MyResourceGroup}

- Run the az group list command to confirm the resource group is deleted.

az group list

# Next Steps

In this tutorial you created an IoT Hub and added the Device Update resource to it. Then you prepared and deployed the new firmware for the dev board and the simulated leaf device.

To learn more about the APIs of the Device Update agent for Azure RTOS, or the Device Update for IoT Hub service, view https://aka.ms/azrtos/adu

To learn more about Azure RTOS and how it works with Azure IoT, view https://azure.com/rtos.